How to Update BIOS from Android platform to Windows platform for PAT-120 Device

Tools Preparing:

1. micro-USB HUB x 1

2. USB Flash Disk x 1

3. USB Keyboard x 1

4. PAT-120 Device x 1

Note: 1. Below is naming rule for PAT-120’s BIOS file.

Please use corresponding PAT-120 BIOS file for BIOS update purpose.

TAxxxxxx.C4A: For PAT-120 Android platform

TAxxxxxx.C46: For PAT-120 Windows 64bit platform

2. Please make sure the battery power is at least 30%

3. The BIOS update process may take few minutes, please do not power off device.

4. Example here is demo how to update BIOS to TA101131.C46 (Windows platform) on PAT-120 device.

The BIOS version might different from the latest version; please contact FAE Engineer to obtain the latest BIOS version.

Steps:

- Prepare a USB Flash drive, then “Format” USB Flash drive to FAT32 format.

2. Unzip “Windows BIOS_0131C46.zip” file, and then copy all BIOS files to USB Flash drive.

3. Connect micro-USB HUB to PAT-120 device, then connect USB Flash drive (with BIOS files) and USB Keyboard to micro-USB HUB.

4. Power on PAT-120 device, then press “DEL” Key to get into BIOS setting.

5. Select to Security -> Security Boot menu, then ensure Secure Boot is select to “Disable”

6. Select to Save & Exit -> Save Changes and Reset, and select “Yes” to save changes,

then the PAT-120 will reboot automatically.

7. When PAT-120 on POST screen, press “F11” to Select boot device to “UEFI: USB 2.0 USB Flash Driver 0.00, Partition 1” (USB Flash drive), and then press “Enter”.

Press “F11”

8. The BIOS update process will start automatically.

Once BIOS update completely, below illustration will show on screen; then please “Power off” PAT-120 device.

Note:  If press “F11” can’t boot into UEFI screen to starting BIOS update process, please repeat Step 7 ~ Step 8

until BIOS update starts. (This is known BIOS issue in this stage, will fix this issue for official release)

9. “Power on” PAT-120 device again, then press “DEL” key to get into BIOS setting.

Check the BIOS Version has changed to “TA101131.C46” to ensure BIOS update process is successful.

10. Press “F9”, then select “Yes” to “Load Optimized Defaults” of BIOS.

11. Press “F10”, then select “Yes” to “Save configuration and reset” of PAT-120 device.

BIOS update finished!

Note.

Due to the BIOS has been change from Android platform to Windows platform for the PAT-120 device,

if the PAT-120 device with Android OS inside the eMMC before, the user might observe below

“OEM1 ACPI table parse error” screen when loading Android system.

(This is normal symptom due to BIOS platform changed)